Karnaugh Map

2023/10/16

B113040045 許育菖

實驗內容

Eercise1

Let F(w, x, y, z) = w'yz' + w'xy + wxz + xyz + wx'y'.

Draw the Karnaugh map and find all the simplest sum-of-products of F.

If the simplest sum-of-products of F are F1, F2 and F3, verify F1

= F2 = F3 by using Verilog Dataflow modeling.

根據題目，可以得到以下的Karnaugh Map：

1

1

1

1

1

1

1

可以發現需要兩格兩格一組來得出sum-of-products。

Let 	F1 = w'yz' + xyz + wy'z + wx'y'

1

1

1

1

1

1

1

F2 = wx'y' + wxz + w'xy + w'yz'

1

1

1

1

1

1

1

F3 = wx'y' + wxz + xyz + w'yz'

1

1

1

1

1

1

1

由實驗結果可以發現：F(w, x, y, z) = F1 = F2 = F3

Exercise2

Draw the Karnaugh map of F1(v, w, x, y, z) = ( 1, 2, 3, 6, 7, 9, 10, 11, 14, 15, 17, 18, 19, 22, 25, 26, 27, 30 ), and find the simplest sum-of-products of this Boolean function.

If the simplest sum-of-products of F1 is F2, verify F1 = F2 by using Verilog Structural level modeling.

Verilog code：

`timescale 1ns / 1ps

module exercise2(v, w, x, y, z, F1, F2);

output F1, F2;

input v, w, x, y, z ;

wire and1, and2, and3, and4, and5, and6, and7, and8, and9, and10;

and(and1, !v, !w, !x, !y, z);

and(and2, !v, !w, !x, y, z);

and(and3, !v, !w, !x, y, !z);

and(and4, !v, !w, x, y, z);

and(and5, !v, !w, x, y, !z);

and(and6, !v, w, x, y, z);

and(and7, !v, w, x, y, !z);

and(and8, !v, w, !x, !y, z);

and(and9, !v, w, !x, y, z);

and(and10, !v, w, !x, y, !z);

wire and11, and12, and13, and14, and15, and16, and17, and18;

and(and11, v, !w, !x, !y, z);

and(and12, v, !w, !x, y, z);

and(and13, v, !w, !x, y, !z);

and(and14, v, !w, x, y, !z);

and(and15, v, w, x, y, !z);

and(and16, v, w, !x, !y, z);

and(and17, v, w, !x, y, z);

and(and18, v, w, !x, y, !z);

or(F1, and1, and2, and3, and4, and5, and6, and7, and8, and9, and10, and11, and12, and13, and14, and15, and16, and17, and18);

wire n1, n2, n3;

and(n1, !x, z);

and(n2, !v, y);

and(n3, y, !z);

or(F2, n1, n2, n3);

endmodule

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

1

Let 		F1 = 所有值為true的格子加起來

將Karnaugh map中值為true的格子八個八個一組可得到F1的sum-of-product，把得到的方程式定義為F2

⟹	F2 = x'z + v'y + yz'

由實驗結果可發現 F1 = F2

Exercise3

Design the nine's complement converter using Karnaugh maps

and don't-care conditions.

Draw the logic diagram of the nine's complement converter and verify the circuit by using Verilog Structural level modeling.

Verilog code：

`timescale 1ns / 1ps

module exercise3(x0, x1, x2, x3, y0, y1, y2, y3);

output y0, y1, y2, y3;

input x0, x1, x2, x3;

//  y0

not(y0, x0);

//  y1

and(y1, 1, x1);

//  y2

wire and1, and2, xor1, xor2;

and(and1, x1, y1);

and(xor1, !x2, and1);

and(and2, !x1, !y1);

and(xor2, x2, and2);

or(y2, xor1, xor2);

//  y3

and(y3, !x3, !x2, !x1);

endmodule

題目要求輸入一個變數，然後輸出它的9補數，意即該數與它的9補數相加起來會等於9，以2進制表示為1 0 0 1。

從y0往y3做處理，並觀察Nine’s-complement table中的規律

我們可以發現：

y0 = (x0)'

y1 = x1

而y2比較不好直接觀察出它的規律，但經過計算後可以得到：

當	x1 = y1 = 1 and x2 = 0

或是

x1 = y1 = 0 and x2 = 1時，y2 = 1。

其餘情況y2 = 0。

最後y3則是當x2, x1, x0 = 0時，y3 = 1，否則y3 = 0。

實驗心得

這次實驗室我們第一次接觸Karnaugh Map，老實說一開始完全不知道如何下手，還去重翻了之前上課的PPT，才總算有點頭緒。在實驗過程中，我對Karnaugh Map有點改觀，原本我以為它是一個麻煩的東西，沒想到使用它來找出sum-of-product還滿方便的，尤其是在知道原本的解的時候。不過這次的第二題好麻煩喔，看完題目發現至少要寫18個node，當下真的還滿心累的。