Adder

2023/11/03

B113040045 許育菖

實驗內容

Exercise1

題目說明

Design and verify the 8-bit ripple-carry adder composed of eight

full-adders by using Verilog Structural level modeling.

推導

Half-Adder:

`timescale 1ns / 1ps

module halfAdder(output S, C, input x, y);

xor(S, x, y);

and(C, x, y);

endmodule

Full-Adder:

`timescale 1ns / 1ps

module fullAdder(output S, C, input x, y, z);

wire S1, C1, C2;

halfAdder HA1 (S1, C1, x, y);

halfAdder HA2 (S, C2, S1, z);

or(C, C2, C1);

endmodule

Verilog code:

`timescale 1ns / 1ps

module exercise1(output[7:0] Sum, output C8, input [7:0] A, B, input C0);

wire C1, C2, C3, C4, C5, C6, C7;

fullAdder    FA0 (Sum[0], C1, A[0], B[0], C0),

FA1 (Sum[1], C2, A[1], B[1], C1),

FA2 (Sum[2], C3, A[2], B[2], C2),

FA3 (Sum[3], C4, A[3], B[3], C3),

FA4 (Sum[4], C5, A[4], B[4], C4),

FA5 (Sum[5], C6, A[5], B[5], C5),

FA6 (Sum[6], C7, A[6], B[6], C6),

FA7 (Sum[7], C8, A[7], B[7], C7);

endmodule

Half-Adder: 	S = x ⊕ y

C = xy

Full-Adder:	Si = (x ⊕ y) ⊕ Ci-1

Ci = (x ⊕ y)z + xy

8-bit ripple-carry = 8個Full-Adder連接起來

Si = (x ⊕ y) ⊕ Ci-1

Ci = (x ⊕ y)z + xy

i ∈ N, 0 < i < 8

結論

Exercise2

題目說明

Design and verify the 4-bit carry-look ahead adder by using

Verilog Structural level modeling.

- carry propagate: Pi = Ai ⊕ Bi, carry generate: Gi = AiBi

- sum: Si = Pi ⊕ Ci, carry: Ci+1 = Gi + PiCi

- C1 = G0+P0C0

- C2 = G1+P1C1 = G1+P1(G0+P0C0) = G1+P1G0+P1P0C0

- C3 = G2+P2C2 = G2+P2G1+P2P1G0+ P2P1P0C0

- C4 = G3+P3C3 = …

推導

Since C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

⇒	 C4 = G3 + P3G2 + P3P2G1 + P3P2P1G0 + P3P2P1P0C0

Verilog code:

timescale 1ns / 1ps

module exercise2(output [3:0] Sum, input [3:0] A, B, output C4, input C0);

wire G0, G1, G2, G3;

and(G0, A[0], B[0]);

and(G1, A[1], B[1]);

and(G2, A[2], B[2]);

and(G3, A[3], B[3]);

wire P0, P1, P2, P3;

xor(P0, A[0], B[0]);

xor(P1, A[1], B[1]);

xor(P2, A[2], B[2]);

xor(P3, A[3], B[3]);

wire C1, C2, C3;

wire and0;

and(and0, P0, C0);

or(C1, G0, and0);

wire and1, and2;

and(and1, P1, G0);

and(and2, P1, P0, C0);

or(C2, G1, and1, and2);

wire and3, and4, and5;

and(and3, P2, G1);

and(and4, P2, P1, G0);

and(and5, P2, P1, P0, C0);

or(C3, G2, and3, and4, and5);

wire and6, and7, and8, and9;

and(and6, P3, G2);

and(and7, P3, P2, G1);

and(and8, P3, P2, P1, G0);

and(and9, P3, P2, P1, P0, C0);

or(C4, G3, and6, and7, and8, and9);

/*

wire and10, and11, and12, and13, and14;

and(and10, P4, G3);

and(and11, P4, P3, G2);

and(and12, P4, P3, P2, G1);

and(and13, P4, P3, P2, P1, G0);

and(and14, P4, P3, P2, P1, P0, C0);

or(C4, G4, and10, and11, and12, and13, and14);

*/

xor(Sum[0], P0, C0);

xor(Sum[1], P1, C1);

xor(Sum[2], P2, C2);

xor(Sum[3], P3, C3);

endmodule

結論

實驗心得

本週的實驗內容為製作加法器。加法器主要分為兩種，一種是ripple-carry adder，另一種則是carry-look ahead adder，兩種的差別在於：ripple adder在進行多位數的加法時，會比look ahead adder花費更多的時間，因為look ahead adder是直接從傳入值得出每個位數加起來的結果，而ripple adder在計算當前位數時，需要先求出前一個位數的值，並將前一位數相加後的結果帶入運算。

雖然說look ahead adder在運算時可以比較節省時間，但在撰寫它的程式邏輯時好麻煩喔，程式碼要寫一大串，果然這世界上沒有甚麼事情是兩全其美的。