Magnitude Comparator,

Decoder and Multiplexer

2023/11/06

B113040045 許育菖

實驗內容

Exercise1

- Design and verify the 4-bit magnitude comparator by using Verilog Structural level modeling.

- Algorithm -> logic

A = A3A2A1A0;	B = B3B2B1B0

A = B if A3=B3, A2=B2, A1=B1 and A1=B1

equality: xi= AiBi+Ai'Bi'

(A=B) = x3x2x1x0

(A>B) = A3B3' + x3A2B2' + x3x2A1B1' + x3x2x1A0B0'

(A<B) = A3'B3 + x3A2'B2 + x3x2A1'B1 + x3x2x1A0'B0

- Implementation

xi = (AiBi' +Ai'Bi)'

Algorithm -> logic

A = A3A2A1A0

B = B3B2B1B0

A=B: if E = 1 		1010, 1010

A>B: if G = 1		1101, 1011 and 0111, 0101

A<B: if L = 1			0111, 1000 and 1001, 1101

Verilog code

`timescale 1ns / 1ps

module exercise1(

output L, G, E,

input [3:0] A, [3:0] B

);

//  =

wire n0, n1, n2, n3;

or(n0, (A[0] & B[0]), ( (!A[0]) & (!B[0]) ) );

or(n1, (A[1] & B[1]), ( (!A[1]) & (!B[1]) ) );

or(n2, (A[2] & B[2]), ( (!A[2]) & (!B[2]) ) );

or(n3, (A[3] & B[3]), ( (!A[3]) & (!B[3]) ) );

and(E, n0, n1, n2, n3);

//  >

wire n4, n5, n6, n7;

and(n4, A[3], (!B[3]));

and(n5, A[2], (!B[2]), n3);

and(n6, A[1], (!B[1]), n3, n2);

and(n7, A[0], (!B[0]), n3, n2, n1);

or(G, n4, n5, n6, n7);

//  <

wire n8, n9, n10, n11;

and(n8, (!A[3]), B[3]);

and(n9, (!A[2]), B[2], n3);

and(n10, (!A[1]), B[1], n3, n2);

and(n11, (!A[0]), B[0], n3, n2, n1);

or(L, n8, n9, n10, n11);

endmodule

Exercise2

Design and verify a 3-to-8 decoder composed of two 2-to-4 de-coders.

  - Dataflow modeling

  - Structural level (Gate-level) modeling

Verilog code

`timescale 1ns / 1ps

module exercise2(

output D0, D1, D2, D3, D4, D5, D6, D7,

input x, y, z

);

decoder A(D0, D1, D2, D3, x, y, (!z));

decoder B(D4, D5, D6, D7, x, y, z);

endmodule

The Decoder

`timescale 1ns / 1ps

module decoder(D0, D1, D2, D3, A, B, E);

output D0, D1, D2, D3;

input A, B, E;

and(D0, (!A), (!B), E);

and(D1, (!A), B, E);

and(D2, A, (!B), E);

and(D3, A, B, E);

endmodule

先建立兩個2-to-4 decoder，接著再把兩個2-to-4 decoder合併形成3-to-8 decoder。

Exercise3

Design and verify the 8-to-1 multiplexer using Verilog HDL

  - Behavioral level modeling

  - Dataflow modeling

  - Structural level (Gate-level) modeling

Verilog code

`timescale 1ns / 1ps

module exercise3(

output reg [2:0] Y,

input [2:0]I0, I1, I2, I3, I4, I5, I6, I7,

input [2:0] S

);

always @ (I0, I1, I2, I3, I4, I5, I6, I7, S)

case (S)

3'b000:     Y = I0;

3'b001:     Y = I1;

3'b010:     Y = I2;

3'b011:     Y = I3;

3'b100:     Y = I4;

3'b101:     Y = I5;

3'b110:     Y = I6;

3'b111:     Y = I7;

endcase

endmodule

實驗心得

此次實驗是我們首次使用Behavioral level modeling來撰寫verilog，我原本以為新的modeling會不太好寫，為此還特地從第三題開始進行實驗，沒想到比想像中的還簡單，三題總共只花費一個多小時就完成了，讓我難得地提早交卷。

這次實驗我在撰寫第二題的時候發生了一個小插曲，題目圖片上的x, y, z沒有按照順序排，我以為是出題者不小心打錯了所有就把x, y, z按順序輸入，沒想到輸出出來的圖形雖然是正確的，但排序完全錯誤，看來我錯怪出題者了，sorry。